#include <WiFi.h>

#include <WebSocketsServer.h>

#include <Wire.h>

#include <MPU6050.h>

const char* ssid = "YOUR_WIFI_NAME";

const char* password = "YOUR_WIFI_PASSWORD";

WebSocketsServer webSocket = WebSocketsServer(81);

MPU6050 mpu;

float xPos = 300;

float yPos = 200;

const char webpage[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<title>ESP32 Motion Pen</title>

</head>

<body>

<canvas id="canvas" width="800" height="500"

style="border:1px solid black;"></canvas>

<script>

var canvas = document.getElementById("canvas");

var ctx = canvas.getContext("2d");

var ws = new WebSocket("ws://" + location.hostname + ":81/");

var x = 300;

var y = 200;

ws.onmessage = function(event) {

var data = JSON.parse(event.data);

x += data.dx;

y += data.dy;

ctx.fillRect(x, y, 3, 3);

};

</script>

</body>

</html>

)rawliteral";

void handleWebSocket(uint8_t num, WStype_t type, uint8_t * payload, size_t length) {

}

void setup() {

Serial.begin(115200);

Wire.begin();

mpu.initialize();

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

}

webSocket.begin();

webSocket.onEvent(handleWebSocket);

Serial.println(WiFi.localIP());

}

void loop() {

webSocket.loop();

int16_t ax, ay, az, gx, gy, gz;

mpu.getMotion6(&ax, &ay, &az, &gx, &gy, &gz);

float dx = ax / 5000.0;

float dy = ay / 5000.0;

String json = "{\"dx\":" + String(dx) + ",\"dy\":" + String(dy) + "}";

webSocket.broadcastTXT(json);

delay(30);

}

#include <WiFi.h>

#include <WebServer.h>

#include <WebSocketsServer.h>

#include <Wire.h>

#include <MPU6050.h>

const char* ssid = "YOUR_WIFI";

const char* password = "YOUR_PASS";

WebServer server(80);

WebSocketsServer webSocket = WebSocketsServer(81);

MPU6050 mpu;

#define BUTTON_PIN 4

// Motion variables

float angleX = 0, angleY = 0;

float gyroAngleX = 0, gyroAngleY = 0;

float filteredX = 0, filteredY = 0;

float sensitivity = 2.5;

unsigned long prevTime;

const char webpage[] PROGMEM = R"rawliteral(

<!DOCTYPE html>

<html>

<head>

<title>Advanced ESP32 Pen</title>

</head>

<body>

<h2>Advanced Motion Pen</h2>

<canvas id="canvas" width="900" height="500"

style="border:1px solid black;"></canvas>

<script>

let canvas = document.getElementById("canvas");

let ctx = canvas.getContext("2d");

let ws = new WebSocket("ws://" + location.hostname + ":81/");

let x = 450;

let y = 250;

ws.onmessage = function(event) {

let data = JSON.parse(event.data);

if(data.draw){

x += data.dx;

y += data.dy;

ctx.fillRect(x, y, 3, 3);

}

};

</script>

</body>

</html>

)rawliteral";

void handleRoot() {

server.send_P(200, "text/html", webpage);

}

void setup() {

Serial.begin(115200);

Wire.begin();

mpu.initialize();

pinMode(BUTTON_PIN, INPUT_PULLUP);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

}

server.on("/", handleRoot);

server.begin();

webSocket.begin();

prevTime = millis();

Serial.println(WiFi.localIP());

}

void loop() {

server.handleClient();

webSocket.loop();

int16_t ax, ay, az, gx, gy, gz;

mpu.getMotion6(&ax, &ay, &az, &gx, &gy, &gz);

unsigned long currentTime = millis();

float dt = (currentTime - prevTime) / 1000.0;

prevTime = currentTime;

// Accelerometer angle

angleX = atan2(ay, az) * 180 / PI;

angleY = atan2(ax, az) * 180 / PI;

// Gyroscope (deg/s)

float gyroXrate = gx / 131.0;

float gyroYrate = gy / 131.0;

gyroAngleX += gyroXrate * dt;

gyroAngleY += gyroYrate * dt;

// Complementary filter

float finalX = 0.98 * gyroAngleX + 0.02 * angleX;

float finalY = 0.98 * gyroAngleY + 0.02 * angleY;

// Smooth output (low-pass)

filteredX = 0.7 * filteredX + 0.3 * finalX;

filteredY = 0.7 * filteredY + 0.3 * finalY;

float dx = filteredX * sensitivity * 0.05;

float dy = filteredY * sensitivity * 0.05;

bool drawState = digitalRead(BUTTON_PIN) == LOW;

String json = "{\"dx\":" + String(dx) +

",\"dy\":" + String(dy) +

",\"draw\":" + (drawState ? "true" : "false") + "}";

webSocket.broadcastTXT(json);

delay(15);

}